Detail Technical Specifications for Multi-Hazard Automatic Sirens

| Audio Emergency Notification System |  |  |
| --- | --- | --- |
|  | Brand/Make | Specified by the Bidder |
|  | Model | Specified by the Bidder |
|  | Country of Origin | Specified by the Bidder |
|  | Alerts | Should support at least 20 configurable alerts |
|  | Triggering | SMS, internet, wireless remote control Dual SIM, 4G or better should be backwards compatible to 3G, Edge and GPRS |
|  | Modem | Support communication via satellite. |
|  | Audio | Pre-recorded voice stored in flash memory in compressed format such as mp3 |
|  | Online configuration | Messages |
|  | Amplifier | Class D, high efficiency |
|  | Power | 150W x 2 channels or higher |
|  | Horn Speaker | Rugged aluminum Horn x 2 * Can be installed facing same direction or opposite direction |
|  | Frequency response: | 160 Hz to 5 KHz |
|  | SPL: | 120 dB |
|  | Power | Battery backed solar power system. Supporting 60 minutes of continuous operation. And recharge by solar panels in 12 hours. |
|  | Online Monitoring | Online monitoring of battery and solar voltage |
|  | SMS Triggering | SMS triggering only from registered numbers |
|  | Weather-proof Enclosure | The Weather-proof enclosure should be at a height of 1 to 2 m, whereas the audio driver |
|  | Mast | Needs to be in a height of 3 to 4 m above the ground. The mast needs to be galvanized. |
|  | Lightning arrester and grounding kit | The top of the mast should have a lightning arrestor made of copper. The lightning arrestor and electrical grounding should have separate ground rods. |
|  | Warning Light | There should be a strobe light that lights up/ flashes when an alert is triggered. |
|  | Environment Conditions: | -20 ̊C to + 50 ̊C |
|  | Warranty | Min. 3 years |
| Wireless Remote Control Should use Nepal unlicensed bands. |  |  |
|  | Wireless communication | Should work in a distance of up to 5km line of sight. Should support triggering of pre-recorded messages, and on demand voice recording and broadcasting. |
|  | Triggering options | The LCD in the remote control should display the health status of the AEWNS, battery voltage, error messages. |
|  | Remote health status | Should have a built in Lithium based battery that will last at least 1 month without charging. |
|  | Built in Lithium based battery | Charger should be included. |
| Central Monitoring and Control Software Record information of each AEWNS, location, contact person etc. Provision to add user |  |  |
|  | Metadata Management | Defined metadata fields. |
|  | Monitoring | Should be able to monitor the health of each AEWNS, including communication status, battery voltage, solar voltage etc. |
|  | Configuration | Online configuration of each AEWNS, authorized mobile numbers for triggering, paired remote control units, upload audio messages etc. |
|  | Triggering | Remote triggering of an AEWNS or a group of AEWNS remotely. |
|  | Audio | The software should support uploading audio files in the server, uploading in the AEWNS, text to speech supporting English and Nepali language using Devanagari fonts. |
|  | CAP Compliant | Should support common alerting protocol, cap alerts can automatically trigger the AEWNS. |
|  | Reporting | Reporting of who, when and which audio message was triggered. |
|  | Maintenance Management System | Supporting tickets and work orders. |
|  | User management | User management with roles and permissions for each users. |